IIoT Workcell - Project Status Report and Next Steps

Overview

A PLC-coordinated robotic workcell demonstrating automated wafer handling across two robot arms and a conveyor. The system was built to replicate an industrial IIoT manufacturing cell in the lab, with an Allen-Bradley PLC serving as the central coordinator.

Cycle Description

A single start command initiates the following sequence:

- Dobot M1 Pro, a 4-axis SCARA with a fork end-effector picks a glass wafer from the staging position and places it in a carrier on the conveyor belt

- Conveyor indexes the carrier a fixed distance to the far station

- myCobot Pro 600, a 6-axis collaborative arm with a pneumatic vacuum cup descends, acquires the wafer, traverses to the drop zone, releases it with a blow-off pulse, and returns to home

The PLC waits for each device to report completion before initiating the next. No timers are used for sequencing.

System Architecture

Hardware

| Component | Model |
| --- | --- |
| PLC | Allen-Bradley Micro850 2080-LC50-24QBB |
| Robot 1 | Dobot M1 Pro |
| Robot 2 | Elephant Robotics myCobot Pro 600 |
| Conveyor | Dobot DT-AC-CB070-02E, stepper with TB6600-class micro step driver |
| Vacuum | SAC40A pneumatic controller, dual 24V solenoid valves |
| Network | 26-port managed switch, all devices on a dedicated 192.168.10.x subnet |

The integration challenge

Neither robot exposes a usable Modbus interface for accepting motion commands. The Pro 600 has no Modbus TCP server at all, its Modbus implementation is RTU-master over RS485, intended for tool-side devices. The M1 Pro has a Modbus server, but its register map does not permit commanding motion in the controller's default mode.

The solution was a translation layer: lightweight Python bridge applications that poll the PLC over Modbus TCP and drive each robot through its native protocol.

Communication

PLC to bridges, Modbus TCP. The Micro850 acts as a Modbus server; both bridges are clients.

Bridges to robots, native protocols. The Pro 600 uses a socket API on port 5001; the M1 Pro uses ports 29999 for control and 30003 for motion.

Register map

| Device | Command | Status |
| --- | --- | --- |
| Pro 600 | 400001 | 400002 |
| M1 Pro | 400011 | 400012 |
| Reserved for expansion | 400021+ |  |

Vacuum valves are mapped as coils at 000001 and 000002.

The handshake

Each device follows a four-state protocol:

- PLC writes a job number to the Command register

- Bridge sets Status to 1 (busy) and begins execution

- Bridge sets Status to 2 (done) on completion

- PLC clears Command to 0; bridge returns Status to 0 (idle)

Completion is detected genuinely rather than estimated. The M1 Pro's Sync() blocks until its motion queue empties; the Pro 600's command_wait_done() behaves equivalently; the conveyor's MC_MoveRelative asserts a Done flag. A fault produces Status 99, which halts the sequence rather than allowing it to continue into a potential collision.

Division of responsibility

The PLC never handles robot coordinates. Its entire vocabulary is "execute job N" and "report status." Taught waypoints reside in the bridge applications and never leave them.

This separation means a waypoint can be re-taught with a single line edit and no PLC modification, and additional jobs are added by extending the bridge's job table without changing the register map.

PLC Program Structure

Ladder logic handles the conveyor axis and vacuum outputs:

- MC_Power holds the PTO axis enabled

- MC_MoveRelative indexes the belt on command

- Two rungs map vacuum coils to physical outputs

Structured Text implements the sequencer, a six-state machine that issues commands, monitors status registers, and advances through the cycle. ST was chosen for the state machine because it expresses six states in roughly twenty readable lines, where equivalent ladder would require extensive comparison and move blocks.

Verification

- Both robots individually stepped through their full trajectories five times each

- Full integrated cycle executed five times consecutively without fault

- Vacuum coil independence verified at the Modbus level

- Fault handling confirmed: a mid-cycle robot timeout correctly halted the sequencer rather than advancing

Planned Development

Sensing

Wafer inventory sensing, detect how many wafers are staged and ready, so the cell knows whether it can begin a cycle and can signal when the supply is exhausted.

End-effector presence detection, confirm that each robot is actually carrying a wafer. On the Pro 600 this can use the vacuum controller's existing feedback outputs (OUT1/OUT2), which are currently unexplored and likely provide a vacuum-present signal. On the M1 Pro's fork, a separate sensor will be required. This converts assumed grip into verified grip and allows the cycle to fault cleanly rather than continuing with a dropped part.

Conveyor position sensing, sensors to start and stop the belt based on actual carrier position rather than a fixed index distance. This removes the dependence on repeatable stepper travel and makes the handoff position self-correcting.

Inspection

Vision system, a camera to inspect wafers and sort acceptable parts from defective ones, introducing a decision point into what is currently a linear sequence. This will require the sequencer to branch, and adds a reject path to the cell.

Process expansion

Laser engraving station, a marking operation positioned mid-conveyor, adding a third processing job to the cycle. The existing register map has deliberate gaps reserved for exactly this kind of addition; the new station receives its own command and status register pair and the same four-state handshake, with no changes required to existing devices.

Supervisory layer

Ignition SCADA dashboard, Ignition reads Modbus TCP natively and will connect to the Micro850 as a device, binding tags directly to the existing command and status registers. Because all device state is already centralized in a contiguous PLC register block, no additional PLC development is required to expose it.

Event logging, a historical record of every cycle, sensor reading, fault, and state transition. This provides cycle-time analysis, fault frequency tracking, and the process traceability expected of a production cell.

Digital Twin - Project Status Report and Next Steps

Simulation side. Revised 10 September 2026 after reading the workcell report above, the full project summary, the twelve Python scripts and the answers returned in the data request. Everything below is verified in simulation unless marked otherwise. Nothing here has yet touched the physical cell.

Overview

A physics simulation of the same workcell, built in Gazebo Harmonic under ROS 2 Jazzy. It reproduces both arms, the conveyor, the carrier and the wafer as rigid bodies with real geometry and real joint limits, and it runs the same handoff sequence the cell runs.

The purpose is a digital shadow: the simulation mirrors the physical cell live, driven by the machine's own signals rather than by a script. The simulated cycle is finished and verified. The connection to the cell is not built yet, but after reading the workcell report it is no longer an open design question - the cell already publishes everything the shadow needs, and the approach is agreed.

Simulated Cycle

One command runs the same stages as the physical cell:

- Dobot M1 Pro slides its fork under a wafer resting in the staging tower, lifts it, turns the wrist ninety degrees during the traverse, enters the carrier across the belt between its two posts, sets the wafer down and withdraws the blade beneath it.

- The conveyor carriage indexes the wafer to the far station.

- myCobot Pro 600 descends onto the wafer, lifts it clear, traverses to the far tower and releases it onto the ledge.

- The carriage returns and the cycle repeats.

The wafer is held by a joint that is made and broken at pick and release rather than by simulated friction. A friction-held payload is unstable at this scale and would have been wrong at any tuning effort.

Simulation Architecture

| Layer | What it is |
| --- | --- |
| Simulator | Gazebo Harmonic, gz-sim 8.11, ODE contacts under DART |
| Middleware | ROS 2 Jazzy with ros2_control |
| Description | one generated URDF: both arms plus the conveyor, 11 joints |
| Control | one controller manager, three joint trajectory controllers |
| Cycle | a single 36-step table shared by the sequencer and the shadow |
| Cameras | six simulated cameras for millimetre inspection and recording |

One description, not three.

Both arms and the conveyor live in a single robot description with a single controller manager. Splitting them into three was tried and abandoned: the Gazebo-side control plugin blocks on the main thread while it waits for its description, so multiple instances deadlock the simulator, and per-instance namespaces are not honoured. Since every device is bolted to the same bench, one kinematic tree is also the physically honest model.

Geometry from the parts, not from eyeballing.

The towers, the carrier and the fork are the actual printed STLs. Positions came from an overhead photograph of the bench, rectified to remove perspective and corrected for camera parallax. Three defects in the vendor's robot description were found and fixed on import, one of which tilted the fork by a degree - enough to drop a wafer edge onto a two-millimetre lip.

Fidelity

- Wafer diameter 127.0 mm against a 128.0 mm step in both towers and in the carrier lip - half a millimetre of radial clearance per side, measured off the STLs rather than assumed.

- The handoff into the carrier was rebuilt to match the cell video frame by frame, including the wrist rotation that turns the fork across the belt.

- The carrier rides the centre of the belt surface, which is not the centre of the conveyor's outline - the motor housing offsets it by thirty millimetres.

- Tall collision meshes are cut into horizontal bands so contact checking stays cheap; without that the simulation ran at half speed whenever an arm passed over a tower.

Verification

- Full cycle runs end to end without fault, repeatedly, and was reviewed and approved against the cell video.

- Through the entire belt traverse the wafer sits on the carrier centre to within a tenth of a millimetre, with no measurable tilt.

- The place into the far tower lands on the nest centre within a few tenths of a millimetre.

- Real-time factor 0.93 to 1.00 across the whole cycle with the graphical window open.

- A six-camera recording of one complete cycle is kept as evidence, alongside close-ups of the pick, the handoff and the place.

- Every fixture's position is checked automatically against the bench rectangle, and the check fails the build if the description and the world disagree.

What the Workcell Report Settles

Seven questions were sent to the cell side. All seven came back, and the full project summary answered a good deal more. The effect on this side is larger than expected, so it is worth recording plainly:

- The PLC speaks Modbus TCP, not EtherNet/IP. The shadow's PLC reader was written against tag reads, on the assumption that named variables would have to be exposed. They do not. The cell already publishes its state as Modbus registers on 192.168.10.10, and two clients already read them. That reader is now a rewrite of one file rather than a negotiation about the PLC program.

- The four-state handshake replaces the step counter that was going to be requested. Each device has a command register and a status register carrying idle, busy, done or fault. That pair already says which job is running and how far it has got. No addition to the PLC program is needed for the shadow to follow the cycle.

- The PLC is the single source of cell state. Confirmed directly: the robots report completion to the PLC over Modbus and the PLC issues the next task. So reading the PLC alone is enough to know what the cell is doing - the arms do not have to be interrogated for the shadow to track the sequence.

- Vacuum is on PLC coils 0 and 1. Readable in the same connection. The grip and the release are directly observable.

- There are no sensors on the cell today. Proximity sensors are planned. Until they exist, carrier position has to be inferred from the conveyor's motion command, exactly as the simulation already does.

- The cycle starts from a button at the PLC, so the shadow has a clean trigger to synchronise a run against.

- The belt currently travels one way only. This is a genuine divergence and is listed under Known Gaps below.

Notes from the Bench Photographs

A set of photographs of the electrical panel adds three things that do not appear in the written notes. Two of them affect how this side connects.

- The controller's catalogue number is 2080-L50E-24QBB. That is what the nameplate reads; the written notes give 2080-LC50-24QBB. It is the same Micro850 family and nothing functional changes, but the plate is the number to quote when looking up a manual or a firmware revision. The same plate also gives series B, firmware 22.011 and a build date of November 2025, none of which is recorded elsewhere.

- The network switch has a three-position mode selector, and one position would break the cell. It is a MokerLink POE-G242GS, and its front panel carries a slider marked Extend(17-24), Default and VLAN(1-22). The VLAN position isolates the numbered ports from one another so each can only reach the uplink. Every device sits on ports one to four, so in that position the bridges would stop seeing the PLC and this simulation would see nothing at all - and it would present as a network fault rather than as a switch setting. The cell works today, which proves the slider is on Default. It is now a pre-flight check.

- The stepper driver's current-limiting resistors are built on a solderless breadboard. The written notes work through the resistor value carefully but never say how it is physically built; the photographs show it. It is the most fragile connection in the cell, and an intermittent contact there produces exactly the symptom the troubleshooting notes already list - works slowly, stalls when driven faster. That matters here because capturing shadow data means running many cycles in succession. If the conveyor misbehaves during a capture session, the breadboard is the first thing to suspect, and the fault belongs to the bench rather than to the model.

Smaller things from the same photographs, all of which make bench work easier: every Ethernet cable is labelled by hand, the switch has around twenty free ports, and the controller's front panel carries a RUN / REM / PRG selector alongside POWER, RUN, FAULT, FORCE and COMM indicators. In the PRG position the program does not execute and every register would simply freeze. The FORCE indicator is a free diagnostic, since a cycle is started today by forcing a variable.

Known Gaps and Divergences

Stated plainly, because they bound what the shadow can currently claim. The first two are new, found by comparing the simulation against the taught waypoints and the answers:

- The simulated belt returns the carriage under power; the real one does not. The cell indexes the carrier one way and it is returned by hand, with reversal planned but not built. The simulated cycle closes its own loop, so the two diverge from the second cycle onward. The simulation should stop returning the carriage until the cell does.

- The simulated fork withdraws by translating; the taught path swings out on J1 alone. In the taught program the arm leaves the carrier by rotating about its base with the other three axes held still - J1 swings from 71.53 to 51.78 degrees, and the radius changes by under a millimetre - and only then lifts vertically, which the notes call the only way to leave the wafer behind. The simulation instead lowers the blade and retreats in a straight line. This matters more than the other divergence: that rotate is the move that stalled the arm and tripped a socket timeout when the speed factor was raised from 15 to 25, so it is the most delicate motion in the cycle and the one where a faithful simulation is worth the most. The simulated path should be re-cut to match it.

- Fixture positions are photo-derived and good to roughly twenty millimetres.

- Neither robot's base rotation has ever been measured; both are inferred from what makes the cycle reachable.

- Belt speed is estimated from video at 70 mm/s. The cell commands a fixed index of distance 170 at velocity 20, found empirically, and the notes now treat formal travel-per-revolution calibration as optional polish because the cell works without it. That is fair for the cell but not for the simulation: until those units are converted to millimetres the simulated carriage and the real one cannot be made to travel the same distance. The conversion is one measurement - mark the belt, run a known distance, measure - and it is worth taking even though the cell no longer needs it.

- No connection to the physical cell exists yet. The shadow runs against simulated devices that replay the ideal cycle, which proves the plumbing and nothing more.

How the Shadow Will Connect

Agreed approach: the laptop joins the same switch, and the simulation reads the same registers and ports that the real hardware uses. The shadow becomes a third Modbus client, and a read-only one. It writes nothing, to any device, ever.

Because the PLC holds the cell state, this can be done at two levels of fidelity, and the first carries no risk to the running cell:

Level 1 - task-level shadow, from the PLC alone.

| What the shadow reads | Source | What it gives |
| --- | --- | --- |
| Holding registers 0 and 1 | Pro 600 command and status | which job, and idle / busy / done / fault |
| Holding registers 10 and 11 | M1 Pro command and status | the same for the SCARA |
| Coils 0 and 1 | Vacuum on and blow-off | the moment the cup grips and releases |

One extra Modbus connection to the PLC, nothing else touched. When the PLC commands job 1 on the SCARA, the simulation runs its own job 1 and the two stay in step, advancing together on the same done signals. This is achievable immediately and is the first milestone.

The sequencer is published in the report in full, which makes this exact rather than approximate.

The six states and their transitions are written out: command the SCARA and wait for done, acknowledge and wait for idle, index the conveyor and wait for the move to complete, command the Pro 600 and wait for done, acknowledge, return to idle. The shadow's own state machine can be written to mirror those six states one for one, watching the same registers for the same transitions. That removes the guesswork the shadow would otherwise need, and it means a change to the cell sequence shows up as a mismatch rather than as silent drift.

Level 2 - joint-level shadow.

The simulated arms follow the real joint angles rather than replaying their own trajectory. This needs a live pose feed from each arm and is where the only real risk sits: the SCARA accepts a single controlling session, and the report is explicit that its own software and the bridge cannot both hold it. Whether a separate read-only feed can coexist with a running bridge has to be tested, not assumed. If it cannot, the clean answer is for the two existing bridges to publish the pose they already read, and the shadow subscribes to that instead of opening its own connection.

One small request that would help a great deal.

The sequencer already keeps a Step variable, and the report describes watching it in the Variable Monitor as the position indicator for the run. If Step were readable over Modbus, the shadow would take the cell's own notion of where it is in the cycle directly, instead of reconstructing it from the command and status pairs.

This is a slightly larger request than it first appears. Step is declared local to the sequencer, alongside Start_Cmd, while everything else the sequencer touches is already global. So it would need promoting to a Global Variable first, and then one row in the Modbus mapping. No logic changes, and nothing else in the program is affected - but it is a program edit and a download, not just a mapping entry.

Planned Development

Connection

- Rewrite the PLC reader for Modbus TCP. Registers 0, 1, 10 and 11 plus coils 0 and 1, read-only, against the existing map. This is the immediate next task and needs nothing from the cell side.

- Bring the simulation up on the cell network. The four devices sit on 192.168.10.x with no gateway, on a 26-port switch with only the first four ports used, so this machine takes a free port and its own address in that range - .5 is already the cell laptop. Three pre-flight checks before anything else: the switch's mode slider on Default, the controller's key switch in RUN, and both command registers reading zero. Then confirm the registers read, and watch the values move during a manual cycle before connecting anything to the simulation.

- Reach Level 1. Drive the simulated cycle from the real command and status registers and confirm the two stay in step across several consecutive cycles.

- Test whether a read-only pose feed is safe, on both arms, with the bridges running. This determines whether Level 2 is direct or goes through the bridges.

Fidelity - and this is where the shared files pay off

- Back-solve the bench layout from the taught waypoints. The two arms touch the same physical carrier, so the same fixture is described in both robot base frames: the SCARA's place point and the Pro 600's pick point differ only by the conveyor index - the notes say plainly that the commanded index is the distance from the SCARA's drop point to the Pro 600's pick point. Together with the known belt direction that is a real geometric constraint on the relative placement of the two bases, far tighter than the photograph, and it is the most promising route to retiring the base-rotation gap without a measurement session. It also works the other way: with the two base frames pinned, the index in millimetres falls out of the same equations.

- Adopt the taught poses as the simulation's waypoints. They are measured numbers from the real machine and they replace several guesses outright, including the rest poses. They also carry detail the simulation currently approximates: the Pro 600 descends 28 mm onto the wafer and 37 mm at the drop, both clean verticals with under a millimetre of lateral drift, and it traverses between them almost flat - 7 mm of lift across a 550 mm swing while the wrist turns 64 degrees, with the wafer held by suction alone. The simulation currently uses a uniform 20 mm final descent and a different traverse profile.

- Re-cut the fork withdrawal to the taught arc, and stop returning the carriage under power, closing the two divergences listed above.

- Take the belt calibration when it is done, and replace the estimated 70 mm/s with the measured travel per index.

Toward a twin

- Record and replay. Capture the register traffic through a physical cycle and drive the simulation from that recording offline. Comparing recorded timing against simulated timing is the first honest measure of how close the model is. One practical note: the Pro 600's clock is over a year out, which will confuse any log correlation until it is set.

- Prediction. Once the model tracks the machine, run it ahead of the machine: check reach, clearance and cycle time for a new waypoint before it runs on hardware. Given that a mistaken pose currently reveals itself by the arm moving, this is the change with the most practical value.

- Simulate the planned additions before they are bought or mounted. The proximity sensors, the grip confirmation and the carrier-position sensing can all be placed in simulation first to decide where they should sit and what they would see. The laser station can be added as a third station in the simulated cycle; note that it speaks serial to the laptop rather than Modbus, so it will not appear in the register map unless it is bridged like the arms.

- Vision. A simulated camera at the inspection point can produce labelled images of good and defective wafers, which allows the sort decision to be developed before the physical camera exists.

Why reading the PLC is the durable choice.

Three changes are already contemplated on the cell side, and none of them disturbs a shadow that reads the PLC. Moving both bridges onto the robots to run under systemd changes where the bridges live but not what the registers say. Putting the SCARA into its Remote Modbus mode could remove one bridge entirely, and the register contract would still be the thing to read. Adding auto-home states at the start of the cycle lengthens the state machine, which the shadow's mirror would need updating to match - and that is exactly the kind of change that should show up as a visible mismatch rather than quiet drift.

Safety note.

The shadow is read-only and cannot affect the cell. It is worth saying that the hardwired E-stop identified as the highest priority on the cell side is also a prerequisite here: capturing shadow data means running the cell repeatedly, and a software stop cannot interrupt a move already in progress.

Open Questions for the Cell Side

Short list now, and none of it blocks the first milestone:

- Can a second, read-only connection read joint positions while the bridges are running, on either arm? If not, would the bridges publish the pose they already read?

- Could Step be mapped to a spare holding register?

- Once the belt is calibrated, what is the travel per index in millimetres and how long does one index take?

- When the belt gains a reverse, will the carriage return automatically at the end of a cycle, or on a separate command?

- Which speed and timing values are the live ones? The written notes and the scripts disagree in a few places - the SCARA speed factor, the Pro 600 speed, and the blow-off pulse each appear with two or three different values. It only matters for comparing simulated cycle time against the real one, but for that it matters exactly.

Repository: the simulation, the cycle table, the shadow package and the verification recordings are version-controlled and licensed under Apache 2.0.