IIoT Workcell — Full Project Summary

For continuing in a new chat. Everything below is verified working unless marked otherwise.

The system

A PLC-coordinated workcell demonstrating a wafer handoff between two robot arms and a conveyor.

Sequence: M1 Pro picks a silicon wafer with a fork end-effector and places it in a carrier on the conveyor → conveyor indexes the carrier to the far end → Pro 600 with a pneumatic vacuum cup picks it off and sets it aside. End of cycle.

Deliverable: rough working sequence demonstrated Friday.

Hardware

| Device | Model | Notes |
| --- | --- | --- |
| PLC | Allen-Bradley Micro850 2080-LC50-24QBB | project version 22, 2 PTO axes, 4 HSC |
| Robot 1 | Elephant Robotics myCobot Pro 600 | Raspberry Pi base, RoboFlow OS |
| Robot 2 | Dobot M1 Pro | 4-axis SCARA, control box with LAN1/LAN2 |
| Conveyor | Dobot DT-AC-CB070-02E | stepper + TB6600-class microstep driver |
| Vacuum | SAC40A pneumatic controller | 2× 24V solenoid valves, 3.5W each |
| Switch | 26-port 10/100/1000 PoE | ports 1=laptop, 2=Pro 600, 3=PLC, 4=M1 Pro |
| Laptop | ASUS ProArt P16 | adapter is Ethernet 4 (second dongle — first was faulty) |

Network

| Device | IP |
| --- | --- |
| Laptop | 192.168.10.5 |
| Micro850 | 192.168.10.10 |
| Pro 600 | 192.168.10.20 |
| M1 Pro | 192.168.10.40 (LAN2) |

Mask 255.255.255.0, no gateway on any device.

Subnet chosen because ASU WiFi is 10.236.x.x (no conflict) and 192.168.1.x is too commonly used by consumer routers.

Register map

| Device | Cmd | Status | Protocol addr |
| --- | --- | --- | --- |
| Pro 600 | 400001 | 400002 | 0, 1 |
| M1 Pro | 400011 | 400012 | 10, 11 |
| Conveyor (reserved) | 400021 | 400022 | 20, 21 |

Status codes: 0 idle, 1 busy, 2 done, 99 fault.

Handshake: PLC writes job number to Cmd → bridge sets Status=1 → runs job → sets Status=2 → PLC clears Cmd to 0 → bridge sets Status=0. Four states, no missed edges.

CCW's Modbus Mapping table uses 6-digit notation (400001), which is protocol address 0 in Python. Verified — no off-by-one.

Architecture: why two bridge scripts

Neither arm can be driven directly by the PLC over Modbus.

Pro 600 — ss -tlnp | grep 502 returned nothing. No Modbus TCP server. RoboFlow's Modbus page is RTU-master-over-RS485 for tool devices only.

M1 Pro — port 502 is open, but a blind scan of holding registers 0–99, coils, and input registers returned all zeros (only discrete input 1 was True). DobotStudio's Modbus panel has a Connect button — it's the robot acting as master to external devices, not a slave.

So both use the same pattern: a Python bridge polls the PLC's holding registers and drives the arm through its native protocol.

Trajectories are still taught in each robot's own software by dragging the arm; the joint values are then pasted into the bridge scripts. Teaching stays, runtime moves.

Worth revisiting after the demo: DobotStudio has a Remote Modbus mode that likely exposes a real writable register map. That would eliminate the M1 Pro bridge entirely and simplify the Ignition integration.

Pro 600 (192.168.10.20)

- Socket API on port 5001, owned by RoboFlow (pid 1051)

- Library: pymycobot, ElephantRobot(ip, 5001) then start_client()

- command_wait_done() blocks until motion completes — this is what makes the handshake honest

- Robot must be powered on via RoboFlow's Configuration Center; reading pose works without servos enabled, commanding motion does not

- Poses need re-teaching — see Next Steps

The RoboFlow "Set IP" button is broken in two ways. It adds the new address without removing the old one, and it never creates a route for the new subnet. Symptom: Destination Host Unreachable even though ip addr looks right.

Fix — edit /etc/dhcpcd.conf directly, don't use the button:

interface eth0

#static ip_address=192.168.1.160/24

#static routers=192.168.1.1

#static domain_name_servers=192.168.1.1

static ip_address=192.168.10.20/24

No routers line, no DNS line. Reboot. This gives both the address and the route, and it persists.

M1 Pro (192.168.10.40)

LAN1 vs LAN2. LAN1 is fixed at 192.168.1.6 and cannot be changed. Only LAN2 is configurable — DobotStudio says so above the IP fields. Cable must be in LAN2. IP changes require a control cabinet restart, not just Apply.

Control mode must be TCP/IP Secondary development. This is the big one. Settings → Remote Control → Current mode. Options are Online, Remote I/O, Remote Modbus, TCP/IP Secondary development.

In Online mode, port 29999 accepts connections but refuses every command with -1,{},CommandName();. Looks like a broken robot; it's just the mode. Switching to TCP/IP Secondary development fixes it immediately.

Corollary: DobotStudio and the bridge cannot both control the arm.

Protocol:

- Port 29999 — control: EnableRobot(), DisableRobot(), ClearError(), SpeedFactor(n), RobotMode(), GetErrorID()

- Port 30003 — motion: MovJ(x,y,z,r), MovL(...), Sync()

- Sync() blocks until the motion queue empties — the completion signal

- Replies are ErrorID,{},CommandName(); — leading 0, is success, -1, is refused

- Requires EnableRobot() on startup and after any E-stop

- SpeedFactor(25) — nothing holds the wafer on the fork, so this is the ceiling

- 4-axis SCARA: J3 is Z in mm, they always match. J1/J2 arm, J4 wrist.

Taught poses (final, verified):

python

M1_HOME = [314.38,  41.65, 157.12, 107.54]   # also serves as approach

M1_P1   = [314.38,  41.65,  70.53, 107.54]   # descend to wafer

M1_P2   = [314.13, 180.89,  70.53, 107.54]   # traverse low

M1_P3   = [314.13, 180.89, 111.93, 107.54]   # lift

M1_P4   = [ 69.49, 389.61, 110.40, 201.70]   # over conveyor

M1_P5   = [ 69.49, 389.61,  75.62, 201.70]   # descend into carrier

M1_P6   = [203.78, 340.41,  75.62, 179.87]   # rotate about base

M1_P7   = [203.78, 340.41, 151.12, 179.87]   # withdraw vertically

Then HOME. Note the P1→P2 traverse happens at Z 70.53 — low. Watch for obstructions.

Conveyor — PTO

Run/stop on a stepper is not simpler than PTO. A stepper needs a pulse train and the PLC is the only source. PTO is the correct answer, not a fallback.

Terminals (fixed, assigned by CCW — not what you'd guess):

- Pulse Output: IO_EM_DO_00 → O-00

- Direction Output: IO_EM_DO_03 → O-03

Once assigned to motion, these cannot be controlled or monitored by the user program. The physical LED still shows pulses — best diagnostic.

Wiring. PLC outputs are 24V sourcing; driver inputs are 5V optocouplers (an LED, not a logic input). Direct 24V destroys them.

O-00 ──[2 kΩ]──► PUL+       PUL− ──► 0V

O-03 ──[2 kΩ]──► DIR+       DIR− ──► 0V

ENA+/ENA− left open (enabled by default)

Driver VCC/GND ──► 24VDC

Driver A+/A−, B+/B− ──► motor phases

Resistor math: (24 − 1.2) ÷ 2000 ≈ 11mA, inside the opto's 8–15mA range and far under the 100mA limit on high-speed outputs (normal outputs are 2A; PTO ones are not). 5V→none, 12V→1k, 24V→2k. 7k gives only 3.3mA — too low, causes sluggish edges that work slow and fail fast.

DIP switches: SW1=ON, SW2=ON, SW3=OFF (200 pulses/rev). SW4=ON, SW5=OFF, SW6=ON (1.0A).

Axis config — four things that block you:

- General page: uncheck all four — Drive Enable Output, Drive Ready Input, In-Position Input, Touch Probe

- Homing page: uncheck Home Switch Input (checked by default on IO_EM_DI_02)

- Limits page: all four unchecked

- Dynamics: defaults are servo-scale and physically impossible (30000 rpm, 50000 mm/sec²). Changed to Max Velocity 50, Max Accel/Decel 500, Jerk 0.

- Motor and Load: Pulses per Revolution 200. Travel per Revolution still 1.0 — placeholder, see Next Steps.

Vacuum — SAC40A

No English documentation exists for this module; behavior determined empirically.

- IN1 and IN2 are 24V sourcing inputs — confirmed by jumpering +24V to IN1, valve actuated. Connects directly to QBB outputs, no resistors, no relay.

- 3.5W per coil ≈ 145mA, well within the 2A normal-output rating

- Chinese labels: 正压 = positive pressure, 负压 = negative pressure. 棕蓝黑白 = brown/blue/black/white (sensor cable legend)

- Both inputs tested as suction — the blow-off function hasn't been located. May be the front rocker switch, or a second output port. Unresolved.

- OUT1/OUT2 unexplored — possibly vacuum-present feedback, would be useful for confirming grip

Wiring as built: Vac_On → _IO_EM_DO_02, Vac_Blow → _IO_EM_DO_01. Note these are reversed from the original plan. Both verified working from ladder.

Grounding

All 24V devices share one negative rail: PLC output common, driver PUL−/DIR− and supply negative, SAC40A GND and 24V−. Different rails means no return path.

Earth ground is separate — it's a safety conductor, not the DC reference. The robots have their own supplies and communicate over transformer-isolated Ethernet; leave their grounds alone.

Use a terminal block for the common rather than daisy-chaining.

Existing ladder (Prog2-POU)

| Rung | Contents |
| --- | --- |
| 1 | MC_Power — EN to left rail, no contacts. AxisIn=Conveyor, Enable/Enable_+/Enable_− all TRUE, Status→Axis_Ready |
| 2 | MC_MoveRelative — AxisIn=Conveyor, Execute=Run_Cmd, Distance=100.0, Velocity=20.0, Accel/Decel=200.0, Jerk=0.0, Done→Move_Done |
| 3 | Vac_On contact → _IO_EM_DO_02 coil |
| 4 | Vac_Blow contact → _IO_EM_DO_01 coil |

Verified state: Axis_Ready True, Active True, Error False, ErrorID 0.

Local variables: Run_Cmd, Axis_Ready, Move_Done, Vac_On, Vac_Blow — all BOOL. Conveyor is a global created by the axis config; don't declare it.

Important: motion function block pins are AxisIn, not Axis. ST failed on parameter names; ladder shows the correct pins. Use ladder.

MC_MoveRelative moves a fixed distance and stops — that's correct behavior, not a bug. Distance ÷ Velocity ≈ seconds. For run-until-stopped you'd need MC_MoveVelocity + MC_Halt, but indexing suits the handshake better since Move_Done is a natural completion signal.

Python scripts (all on laptop, Documents\Applied Project\Python Scripts)

| File | Purpose |
| --- | --- |
| Bridge.py | Pro 600 bridge — polls reg 0, drives via pymycobot |
| m1_bridge.py | M1 Pro bridge — polls reg 10, raw sockets to 29999/30003 |
| send_cmd.py | py send_cmd.py <register> <value> — stands in for the PLC |
| Modbus.Test.py | register read/write verification |
| Cobot_Test.py | Pro 600 pose read |
| dobot_scan.py | M1 Pro register scan |

python is not on PATH. Use py, or the full path C:\Users\user\AppData\Local\Python\pythoncore-3.14-64\python.exe. Run send_cmd.py from the terminal, not VS Code's Run button — the Run button passes no arguments and throws IndexError.

pymodbus pinned to 3.6.9 (the API changed across 3.x). Also installed: pymycobot.

Debugging reference

Network

| Symptom | Cause |
| --- | --- |
| Ping shows "General failure" mixed with timeouts | Local adapter unstable. Self-ping to your own IP should never drop packets — if it does, the dongle is bad. The original wch.cn adapter had no Power Management tab and was replaced. |
| "Destination Host Unreachable" from the Pro 600 | Missing route. ip route must show 192.168.10.0/24 dev eth0. Caused by RoboFlow's Set IP button. |
| Micro850 shows random 169.254.x.x addresses | DHCP with no server. RSLinx caches every one — five stale ghosts under AB_ETHIP-2. |
| CCW "Save to Controller" appears to work but doesn't apply | Use the full route: Ctrl+S → Disconnect (Shift+F7) → Build → Connect → Download → power cycle. Build/Download/Upload are greyed while connected — that's normal. |

Conveyor

| Symptom | Cause |
| --- | --- |
| Motor silent, no holding torque | Driver unpowered, or ENA pulled low |
| Buzzes, doesn't rotate | Phase pairs wrong — swap A− and B+ |
| Holds position, won't turn | No pulses. Check resistors, O-00, and whether the O-00 LED flickers |
| Axis_Ready stays False | Axis config — a limit or drive-ready input still enabled |
| Works slow, stalls faster | Resistor too large, or velocity beyond the torque curve |

Identify motor phases with a multimeter, motor disconnected: same phase reads a few ohms, different phases read open. Never disconnect a stepper while the driver is energized — inductive spike with nowhere to go.

Robots

| Symptom | Cause |
| --- | --- |
| M1 Pro returns -1,{},Cmd(); to everything | Wrong control mode. Must be TCP/IP Secondary development. |
| M1 Pro unreachable after IP change | Cable in LAN1, or cabinet not restarted |
| Pro 600 reads pose but won't move | Not powered on via RoboFlow Configuration Center |

Micro850 USB access: node 16 under AB_VBP-1 in the Connection Browser. Device Manager shows "Rockwell Automation USB CIP Device". Upload from the controller rather than picking from the catalog — gets the firmware revision right automatically.

Next steps

1. Re-teach the Pro 600 poses. The existing HOME/APPROACH/TOUCH were taught with a bare flange. The vacuum cup extends the tool, so the arm will stop short by the cup's length. Re-teach at the actual pick point on the belt, cup fitted. Use the capture script pointed at 192.168.10.20.

2. Calibrate the belt. Travel per Revolution is still 1.0, so "Distance 100" means 100 motor revolutions, not 100mm. Mark the belt, run Distance 100, measure travel, set Travel per Revolution = measured ÷ 100. Then command the actual distance from the M1 Pro's drop point to the Pro 600's pick point.

3. Write the sequencer ladder. Replaces send_cmd.py. Structure:

| Step | Action | Waits for |
| --- | --- | --- |
| 0 | idle | start button |
| 1 | write 1 to Cmd_Word_2 (M1 Pro) | Status_Word_2 = 2 |
| 2 | clear, trigger Run_Cmd (conveyor) | Move_Done |
| 3 | Vac_On, write 1 to Cmd_Word (Pro 600) | Status_Word = 2 |
| 4 | Vac_On off, clear, return to idle | — |

Open design question: should the vacuum be controlled by the ladder or by the Pro 600 bridge? The bridge knows exactly when the arm is at the pick pose, but the valves are on PLC outputs, so it would need to write a Modbus coil. Simpler for Friday: let the ladder handle it.

4. Safety layer. Hardwired E-stop into a proper safety relay, plus a hardwired "robot in motion" input separate from the Modbus status word. Modbus TCP is not a safety protocol. Worth doing before calling the cell finished.

Deferred past Friday

- Move both bridges to their respective robots and run under systemd so the cell works without a laptop. Change ROBOT_IP to 127.0.0.1, pip install pymodbus pymycobot on each.

- Investigate M1 Pro Remote Modbus mode — could eliminate one bridge entirely.

- Ignition SCADA interface. Ignition reads Modbus TCP natively and would connect to the Micro850 as a device, binding tags to the same registers. This is why the register map has deliberate gaps at 400011 and 400021 — a tidy contiguous block is much nicer to build screens against.

- Locate the SAC40A blow-off function if wafer release proves unreliable.

- Wire OUT1/OUT2 from the SAC40A as a vacuum-present input.

Housekeeping

- CCW project is still named Project5 — rename to something like Workcell_Base and save a known-good copy

- Stale persistent route on the laptop: route delete 0.0.0.0 mask 0.0.0.0 192.168.3.4

- Pro 600's clock is wrong by over a year (shows 2025/6/3) — will confuse bridge logs later

- Back up /etc/dhcpcd.conf on the Pro 600: sudo cp /etc/dhcpcd.conf /etc/dhcpcd.conf.working

- Save both bridge scripts outside the working folder

Summary Addendum — Taught Waypoints & Sequencing Decision

Pro 600 poses (192.168.10.20) — final, with vacuum cup fitted

python

PRO600_HOME      = [ -81.287, -102.418, 130.242, -118.125, -89.736,  -2.109]

PRO600_APPROACH  = [-105.256,  -54.941,  90.283, -124.980, -89.561, -25.928]

PRO600_PICK      = [-105.255,  -50.809,  91.498, -131.045, -89.561, -25.928]

PRO600_DROP_OVER = [ -41.470,  -68.082, 109.797, -132.275, -89.824,  38.320]

PRO600_DROP      = [ -41.470,  -61.617, 111.499, -140.361, -89.824,  38.320]

| Pose | Cartesian (X, Y, Z) | Notes |
| --- | --- | --- |
| HOME | 150.2, -254.9, 261.8 | parked clear of the belt |
| APPROACH | -13.3, -467.0, 193.2 | hovering over wafer on conveyor |
| PICK | -13.5, -467.9, 165.2 | cup contacts wafer — Vac_On here |
| DROP_OVER | 363.2, -174.5, 200.4 | hovering over drop zone |
| DROP | 363.1, -174.4, 163.5 | Vac_On off + Vac_Blow 2s |

Sequence: HOME → APPROACH → PICK → [Vac_On] → APPROACH → DROP_OVER → DROP → [Vac off, Blow 2s] → DROP_OVER → HOME

APPROACH and DROP_OVER are each used twice, so only five poses cover a nine-move cycle.

Descents are clean verticals: 28mm at the pick, 37mm at the drop, with X/Y drift under 1mm in both cases.

Two things to watch:

- The traverse from APPROACH to DROP_OVER is nearly flat — Z 193.2 → 200.4, only 7mm of lift across a 550mm swing, with J6 rotating 64° on the way. Check nothing sits in that path. Add an intermediate lift if it does.

- Speed matters here more than anywhere. The wafer is held by suction alone across that whole traverse.

M1 Pro poses (192.168.10.40) — final

python

M1_HOME = [314.38,  41.65, 157.12, 107.54]

M1_P1   = [314.38,  41.65,  70.53, 107.54]

M1_P2   = [314.13, 180.89,  70.53, 107.54]

M1_P3   = [314.13, 180.89, 111.93, 107.54]

M1_P4   = [ 69.49, 389.61, 110.40, 201.70]

M1_P5   = [ 69.49, 389.61,  75.62, 201.70]

M1_P6   = [203.78, 340.41,  75.62, 179.87]

M1_P7   = [203.78, 340.41, 151.12, 179.87]

| Pose | Action |
| --- | --- |
| HOME | parked; also serves as the approach above P1 |
| P1 | descend to wafer (86.6mm drop) |
| P2 | traverse 139mm in Y at low height — watch for obstructions |
| P3 | lift |
| P4 | over conveyor (large swing, R rotates 107.54 → 201.70) |
| P5 | descend into carrier |
| P6 | rotate about base (J2/J3/J4 unchanged, J1 swings 71.53 → 51.78) |
| P7 | withdraw vertically — the only way to leave the wafer behind |

Then HOME.

Sequencing decision: handshake, not timers

Considered timing each stage and firing the next on a stopwatch. Rejected — every device already reports completion:

- M1 Pro: Sync() blocks until the motion queue empties

- Pro 600: command_wait_done() blocks until motion completes

- Conveyor: MC_MoveRelative sets Move_Done

Timers would break silently on any variation, need retuning after every pose or speed change, and mask faults that the handshake surfaces.

Timers are used only for:

- The 2-second blow-off (no sensor, so a timer is correct)

- A 0.5s settle after vacuum engages, before lifting

- A watchdog — fault the sequence if a step exceeds ~60s

Vacuum control: bridge, not ladder

The Pro 600 bridge drives the vacuum directly via Modbus coils, because it knows exactly when the arm reaches PICK and DROP. The ladder can't know that.

Requires: promote Vac_On and Vac_Blow to global variables, add two rows to CCW's Modbus Mapping as coils, download.

Bridge writes plc.write_coil(VAC_ON_COIL, True) at PICK, and at DROP writes Vac_On False, Vac_Blow True, sleeps 2s, Vac_Blow False.

Open item: does the blow-off actually blow?

Both IN1 and IN2 tested as suction earlier. The 正压 (positive pressure) function was never located.

If Vac_Blow also produces vacuum, energizing it after release would re-grab the wafer.

Test before wiring it into the sequence: hold something on the cup with Vac_On, switch to Vac_Blow, see whether it releases or holds. The front rocker switch on the SAC40A may select between pressure modes.

Fallback if there's no blow: cut the vacuum and pause. Less reliable for a wafer, but workable.

Where things stand

| Item | State |
| --- | --- |
| Network, 4 devices | done |
| PLC Modbus registers, both arms | done |
| Both bridges, handshake verified | done |
| Conveyor PTO, wiring + motion | done |
| Vacuum valves from ladder | done |
| M1 Pro poses | done |
| Pro 600 poses | done |
| Vac coils in Modbus mapping | next |
| Blow-off verification | next |
| Belt calibration | next |
| Sequencer ladder | next |

Good progress — you're two days ahead of where the schedule needed you.

Next

Four items left, roughly 2–3 hours total. I'd do them in this order:

- Blow-off test — five minutes, and it determines whether the drop sequence needs rethinking

- Belt calibration — mark, run Distance 100, measure, set Travel per Revolution

- Vac coils — two rows in Modbus Mapping, then update both bridge scripts with the new poses

- Sequencer ladder — the four-step state machine

Session 3 — Full Cycle Integration

Status: the complete three-device sequence works, 5/5 successful runs

What the cycle does

One button press runs the whole thing:

- M1 Pro picks a silicon wafer with a fork end-effector, carries it across, and places it in the red carrier on the conveyor belt

- Conveyor indexes the carrier to the far end

- Pro 600 descends with a pneumatic vacuum cup, picks the wafer off the carrier, traverses to the drop zone, releases with a blow-off pulse, and returns home

No timers coordinate this. Each device reports its own completion and the PLC waits for that report.

The sequencer — how the PLC coordinates three devices

A Structured Text program (sequencer) runs a six-state machine. This is the piece that turns three independently-working devices into a cell.

CASE Step OF

0:  (* idle - wait for start *)

IF Start_Cmd THEN

Step := 1;

END_IF;

1:  (* M1 Pro: run transfer *)

Cmd_Word_2 := 1;

IF Status_Word_2 = 2 THEN

Step := 2;

END_IF;

2:  (* acknowledge M1 Pro *)

Cmd_Word_2 := 0;

IF Status_Word_2 = 0 THEN

Step := 3;

END_IF;

3:  (* conveyor: index 170 units *)

Run_Cmd := TRUE;

IF Move_Done THEN

Step := 4;

END_IF;

4:  (* Pro 600: pick and place *)

Run_Cmd := FALSE;

Cmd_Word := 1;

IF Status_Word = 2 THEN

Step := 5;

END_IF;

5:  (* acknowledge Pro 600, cycle complete *)

Cmd_Word := 0;

IF Status_Word = 0 THEN

Start_Cmd := FALSE;

Step := 0;

END_IF;

END_CASE;

Why ST rather than ladder here. ST failed earlier on the motion blocks because parameter names differ by CCW version (AxisIn, not Axis). A state machine has no function blocks — just comparisons and assignments — so none of that applies. Six states in ST is about twenty readable lines; in ladder it would be a screen full of comparison and move blocks.

Why each device gets two states. Steps 1 and 2 are the two halves of the handshake. Step 1 issues the command and waits for "done"; step 2 acknowledges by clearing the command and waits for the bridge to return to idle. Without the acknowledge, the bridge would see a stale command still sitting in the register and immediately re-run the job.

Why the conveyor only needs one state. MC_MoveRelative fires on the rising edge of Execute and sets Move_Done when finished. Holding Run_Cmd high through step 3 is harmless — the edge already happened on entry. Step 4 drops it so the next cycle can trigger again.

Variables: Step (INT) and Start_Cmd (BOOL). Everything else the sequencer touches must be global — Cmd_Word, Status_Word, Cmd_Word_2, Status_Word_2, Run_Cmd, Move_Done.

Starting the cycle — and a CCW quirk

Start_Cmd cannot be forced directly. Attempting it gives:

An error occurred while writing to :Controller.Micro850.Micro850.sequencer.Start_Cmd

CCW refuses external writes to a variable the program assigns — and step 5 sets Start_Cmd := FALSE.

Workaround: a ladder rung in Prog2 with a Manual_Start contact driving the Start_Cmd coil. Force Manual_Start instead; nothing writes it, so nothing refuses the write.

To re-run: force Manual_Start False, then True. The transition is what step 0 needs.

Better fix, not yet done: wire a momentary pushbutton to a spare digital input and use its contact directly. Must be momentary — a maintained switch would restart the cycle the instant step 5 clears Start_Cmd.

Vacuum control via Modbus coils

The Pro 600 bridge drives the vacuum directly rather than the ladder, because only the bridge knows when the arm has actually reached PICK and DROP.

Setup: Vac_On and Vac_Blow promoted to Global Variables and added to CCW's Modbus Mapping as coils at addresses 000001 and 000002 — protocol addresses 0 and 1 in Python. Coils are a separate address space from holding registers, so there's no conflict with Cmd_Word at 400001.

Coil independence verified:

python

plc.write_coil(0, True)

print(plc.read_coils(0, count=2).bits[:2])   # [True, False]

plc.write_coil(0, False)

plc.write_coil(1, True)

print(plc.read_coils(0, count=2).bits[:2])   # [False, True]

Both prints came back correct, ruling out coil-packing problems where writing one bit clobbers its neighbour.

Ladder rungs 3 and 4 map the globals to physical outputs: Vac_On → _IO_EM_DO_02, Vac_Blow → _IO_EM_DO_01. Note these are reversed from the original plan.

The vacuum diagnostic

Reported symptom: blow-off appeared to fire mid-traverse, dropping the wafer before the drop zone.

Investigation: added a vac_state() helper printing both coil states at each stage of the job. Clean trace:

[start] Vac_On=False  Vac_Blow=False

-> over wafer

[at approach] Vac_On=False  Vac_Blow=False

-> down to wafer

[after vac on] Vac_On=True  Vac_Blow=False

-> lift

[after lift] Vac_On=True  Vac_Blow=False

-> traverse

[after traverse] Vac_On=True  Vac_Blow=False

-> down to drop

[at drop] Vac_On=True  Vac_Blow=False

[after blow] Vac_On=False  Vac_Blow=False

-> lift

-> home

Vacuum holds through lift, traverse, and descent exactly as designed. Repeat runs were all clean.

Most likely original cause: a leftover coil state from a run that faulted or was Ctrl+C'd mid-cycle, leaving Vac_Blow energized. The next run then looked like premature blow-off when actually the valve had never switched off.

The [start] and [startup] prints now catch this — any run beginning with Vac_Blow=True identifies it immediately.

Recommended guard (not yet added) so an interrupted run can't leave the vacuum on:

python

if __name__ == "__main__":

plc_cleanup = ModbusTcpClient(PLC_IP, port=502)

try:

main()

except KeyboardInterrupt:

print("\nstopped")

finally:

if plc_cleanup.connect():

plc_cleanup.write_coil(VAC_ON, False)

plc_cleanup.write_coil(VAC_BLOW, False)

plc_cleanup.close()

Timing values, tuned

| Parameter | Value | Reasoning |
| --- | --- | --- |
| Vacuum settle at PICK | 1.5s (was 0.5) | cup needs time to seal and build vacuum before lifting |
| Blow-off at DROP | 0.5s (was 2.0) | enough to break the seal; longer risks blowing the wafer out of position after it lands |
| Pro 600 speed | 500 |  |
| M1 Pro SpeedFactor | 15 (was 25) | see below |
| Conveyor Distance | 170.0 at Velocity 20.0 | empirically correct — formal Travel-per-Revolution calibration is now optional polish |

The M1 Pro timeout, and what it taught us

Symptom:

command 1: wafer transfer

-> descend to wafer

-> traverse

-> lift

-> over conveyor

-> descend into carrier

-> rotate

FAULT: timed out

This was a socket timeout, not a robot fault. The move socket has timeout=60 and Sync() didn't reply within that window — the arm stalled during the rotate move (P5 → P6).

The handshake behaved correctly. The bridge caught the exception, wrote FAULT (99) to Status_Word_2, and waited. The sequencer only advances on Status = 2, so it held at Step 1 rather than blundering on to start the conveyor while the arm was stuck in the carrier. That's the design working — and it's worth pointing out on Friday.

Cause: speed. m1_step.py used SpeedFactor 15 during manual testing; the bridge was set to 25. A move that's comfortable at 15% tripped something at 25%.

Fix: bridge dropped to SPEED = 15.

Recovery procedure when a bridge faults:

- Ctrl+C both bridges

- In the Variable Monitor, force to zero: Step, Cmd_Word, Cmd_Word_2, Manual_Start — and clear both vacuum coils

- Clear the M1 Pro fault:

python

import socket

d = socket.create_connection(("192.168.10.40", 29999), timeout=10)

d.sendall(b"ClearError()\n"); print(d.recv(1024))

d.sendall(b"EnableRobot()\n"); print(d.recv(1024))

Or use m1_step.py, which has a c command for this.
4. Step the arm back to HOME with m1_step.py

Operating procedure

Two separate terminals — each bridge is a blocking loop, so one would prevent the other from starting. In VS Code use the + button, or the split-pane icon to watch both at once.

Terminal 1:  py Bridge.py       → wait for "waiting for commands..."

Terminal 2:  py m1_bridge.py    → wait for "waiting for commands..."

Then force Manual_Start in CCW.

Pre-flight checklist:

- M1 Pro parked at HOME. Otherwise the first move takes an unpredictable path from wherever it happens to be sitting. This is a real gotcha — worth automating.

- M1 Pro in TCP/IP Secondary development mode, DobotStudio disconnected. Easy to forget after teaching poses; symptom is -1 on every command.

- Pro 600 powered on through RoboFlow's Configuration Center.

- Step = 0 and both command registers 0 before starting the bridges. If Cmd_Word_2 is left at 1, the M1 Pro will start moving the instant its bridge connects.

Watch Step in the Variable Monitor during the run. It's the position indicator — if something stalls, Step tells you which handoff failed and the corresponding status word tells you why.

Understanding the architecture: where the coordinates live

Worth being explicit, since it came up:

The PLC never sees a joint angle. Poses live in the bridge scripts and never leave them.

PLC writes Cmd_Word_2 := 1

↓

Bridge reads "1", looks up JOBS[1] → job_transfer()

↓

Bridge sends MovJ(314.38, 41.65, ...) to the arm

↓

Bridge writes Status_Word_2 := 2 when Sync() returns

The PLC's entire vocabulary is "run job N" and "are you finished." It doesn't know what job 1 involves, how many waypoints it has, or where they are.

That split is deliberate. Coordinates are robot-domain data the PLC has no use for. It also means re-teaching a pose is a one-line Python edit with no PLC change, and adding a second job is just adding JOBS[2] — the PLC writes 2 instead of 1 and nothing else changes.

It's also why Ignition will be straightforward later: it reads the same command and status registers from the PLC and never needs to know about robot kinematics either.

Still outstanding

1. Hardwired E-stop — highest priority. Both arms have E-stop inputs. A mushroom-head switch through a safety relay into both, plus a contact dropping the conveyor driver's enable.

This matters because the software stop cannot interrupt a move in progress — the bridge is blocked inside Sync() or command_wait_done() and won't check Modbus again until the arm arrives. Modbus TCP is not a safety protocol and neither is a Python script. There will be people standing near two moving arms on Friday.

2. Stop_Cmd software abort. Drafted, not added. Goes before the CASE:

IF Stop_Cmd THEN

Cmd_Word   := 0;

Cmd_Word_2 := 0;

Run_Cmd    := FALSE;

Start_Cmd  := FALSE;

Step       := 0;

Stop_Cmd   := FALSE;

ELSE

CASE Step OF

...

END_CASE;

END_IF;

Clears both command registers and resets to idle; the bridges see their commands go to 0 and return to idle on their own. Won't stop an arm already in motion — that's what the E-stop is for.

3. Physical start button. Momentary switch on a spare input, replacing the forced variable.

4. Auto-home at cycle start. Both bridges already have JOBS[2] for going home. Two extra sequencer states would send both arms home before the cycle begins, removing the "remember to park the M1 Pro" requirement. Worth doing — that's exactly the sort of thing that goes wrong under pressure.

5. Interrupted-run vacuum guard. The finally block above.

6. Housekeeping. CCW project still named Project5. Save known-good copies of both bridge scripts and the taught poses outside the working folder.