Wafer cell · digital shadow · bring-up

What I need from the PLC side

A read-only list. Everything here is either a number to look up or a screenshot to send — except one item, which may need a small change to the PLC program.

Cell: Dobot M1 Pro + myCobot Pro 600 + conveyor      PLC: Micro800 family, programmed in CCW

| Answered by: | Date: |
| --- | --- |

Start here

The message to send him

If you only send one thing, send this. Everything after it is backup detail for when he asks “what do you mean?”

Hi — I'm building a simulation of our wafer cell that mirrors the real machine on screen while it runs. It only reads. It never sends a command to the PLC or either robot. To hook it up I need a few things from your side. None of it is urgent, and none of it changes your program except possibly item 3. 1. The exact PLC model. It's printed on the front of the controller — something like 2080-LC50-24QWB. A photo of the label is perfect. 2. Its IP address and subnet mask. Also: is the PLC plugged into a network switch, or do you only connect a USB cable when you program it? 3. The Global Variables list from your CCW project. Open the project, find Global Variables in the tree on the left, and send me a screenshot of the table — I need the names and the data types. One important thing: if the belt and vacuum flags only exist as local variables inside the program, I can't read them over the network. If that's the case, could you add Global Variables that mirror them? 4. Which variable is which. Which one turns the belt on, which one sets the belt direction, which one turns the vacuum on, and is there one that holds a step or state number for the cycle? Exact spelling matters. 5. Does the belt ever run backwards, and roughly how many seconds does it run in each direction per cycle? 6. Permission: is it OK if I plug my laptop into the same switch and read values while the cell is running? Read only, nothing written. If it's easier, just send me the whole CCW project archive (the .ccwarc file) and I'll pull the details out myself.

Part 1

The PLC — his home turf

Take these in order. Each one only makes sense once the one before it is answered.

BLOCKER

1.1  Which controller is it, exactly?

SAY IT LIKE THIS “What's the exact model number on the front of the PLC? Something like 2080-LC50-24QWB. A photo of the label works.”

Why it comes first: not every controller in this family has an Ethernet port. If this one doesn't, everything else on this page changes, and we'd need a different way to get the signals out.

The number also tells me how many inputs and outputs it has, which is a sanity check against the signal list in 1.4.

BLOCKER

1.2  What is its network address?

SAY IT LIKE THIS “What's the PLC's IP address and subnet mask? And when you program it, do you plug in an Ethernet cable or a USB cable?”

If he says USB: that's a real possibility, and it means the Ethernet port may never have been configured. It's not a problem — it just turns this from a question into a small setup task for him, and it's better to find out now than on demo day.

Where to look

1.  Some controllers show the address on a small display on the front.

2.  Otherwise it's in the CCW project, in the controller's settings under the Ethernet page.

3.  Menu names shift a little between CCW versions — a screenshot of whatever he finds is fine.

BLOCKER   MAY NEED A CHANGE

1.3  The Global Variables list

SAY IT LIKE THIS “Can you open Global Variables in the CCW project and send me a screenshot of the table — the names and the data types? And can you check that the belt and vacuum flags are Global, not local to the program?”

This is the one item that may need him to edit his program. On this family of controller, only Global Variables can be read from the network. A variable that lives inside a program is invisible from outside, no matter what I do at my end.

If his flags are local, the fix is small and safe: add a Global Variable for each one and assign the local value to it in one rung. It doesn't change how the machine behaves.

Where to look

1.  Open the project in Connected Components Workbench.

2.  In the tree on the left, under the controller, there's a Global Variables entry.

3.  Open it — it's a table with Name, Data Type, Initial Value and Comment.

4.  A screenshot is enough. The whole project archive is better.

BLOCKER

1.4  Which variable does what

SAY IT LIKE THIS “For each of these, what's the exact variable name and its type — belt on/off, belt direction, vacuum on/off, and a step or state number if the program keeps one?”

Spelling and capitals have to be exact, the way they're written in the table. My side asks for a variable by name; a near miss just returns nothing.

The step number is the valuable one. If his program tracks where it is in the cycle as a number, the simulation can follow the real machine step by step. Without it I have to guess the sequence from the belt and vacuum turning on and off — that works, but it's guesswork.

What I'm currently guessing: the names in my config today are placeholders — BeltRun, VacuumOn, CycleStep. Almost certainly wrong. Correcting a wrong guess is faster than answering a blank question, so it's worth showing him this line.

HELPFUL

1.5  A look at the program itself

SAY IT LIKE THIS “Could you send a screenshot of the main ladder routine, or the whole project archive? I want to see the order the cycle runs in.”

The simulation has its own list of cycle steps that I built from the videos. Seeing his logic lets me line the two up so they use the same names and the same order — that's the difference between a simulation that looks right and one that genuinely tracks the machine.

HELPFUL

1.6  Timings and direction

SAY IT LIKE THIS “How many seconds does the belt run each time? Does it ever go backwards? Is there a pause partway along? And are there any sensors on the belt, or is it all on timers?”

There are no belt sensors as far as I can tell, so my simulation works out where the holder is by counting time from when the belt starts. If that's right, I need his timer values. If there is a sensor, that's much better and I'd rather read it.

BLOCKER

1.7  Permission to connect

SAY IT LIKE THIS “Is it alright if I plug my laptop into the same switch and read values while the cell is running? Read only — I won't write anything or go online with the controller.”

Worth asking plainly rather than assuming. A PLC programmer's instinct when someone connects to their controller is that the program is about to be changed. Saying “read only, I'm not going online with it” up front removes that worry.

Part 2

The two robots

He may not own these, but he'll know who does, or the numbers may be on the teach pendants.

BLOCKER

2.1  The addresses of both arms

SAY IT LIKE THIS “What IP addresses do the Dobot M1 Pro and the myCobot Pro 600 have? They're usually shown on the pendant's network or settings screen.”

My current guesses are the vendor defaults — 192.168.1.6 for the Dobot and 192.168.1.159 for the Pro 600. Real benches rarely keep those.

For the Pro 600 there's a second part: its remote-control server has to be switched on for anything to read from it. Ask whether it's enabled, and if he doesn't know, that's fine — we can find out by trying.

BLOCKER

2.2  The one-joint-at-a-time test

SAY IT LIKE THIS “Can we jog one joint at a time and write down what the pendant says? Move joint 1 on its own, note the number, then joint 2, and so on. Ten minutes for both arms.”

This is the least obvious ask and one of the most valuable. The real robot and my model may count the same joint in opposite directions, or from a different zero point. Without this the simulated arm mirrors the real one but bends the wrong way.

It's a five-minute job per arm and it removes a whole class of confusing errors later. Doing it together is much faster than describing it over messages.

HELPFUL

2.3  The parked position

SAY IT LIKE THIS “When both arms are sitting at their start-of-cycle position, can you photograph the joint numbers on each pendant?”

My rest poses were calculated, not measured from the real machine. Real numbers would replace a guess I've been carrying since the beginning.

Part 3

The bench network

Small, quick, and it decides whether any of the above can actually be read on the day.

BLOCKER

3.1  Can I get on the same network?

SAY IT LIKE THIS “Is there a network switch at the bench with a spare port? What range are the addresses in — do they all start 192.168.1.something? And is anything else on that network, like a company router?”

If the three devices sit on an isolated switch, I plug in, give my laptop an address in the same range, and everything works. If they're on the building network, there may be rules about what can be plugged in — better to know before carrying a laptop over.

Part 4

Twenty minutes together beats twenty messages

If he's willing to sit at the bench with you once, this replaces most of the back-and-forth.

What we'd do in that session

1.  Plug my laptop into the switch and confirm I can see all three devices.

2.  Read one variable from the PLC while he watches — proves the whole path works and reassures him nothing is being written.

3.  Run one cycle while I record everything the three devices report.

4.  Do the one-joint-at-a-time test on both arms.

That recording alone would let me finish the shadow without him being present again.

Sheet

Answer sheet

Fill in the right-hand column. The middle column is what I'm assuming today — most of it is probably wrong, and that's fine.

| What | My guess today | Actual |
| --- | --- | --- |
| PLC model | Micro850 family |  |
| PLC address | 192.168.1.20 |  |
| Belt on/off variable | BeltRun (BOOL) |  |
| Belt direction variable | none — belt never reverses |  |
| Vacuum variable | VacuumOn (BOOL) |  |
| Step / state number | CycleStep — may not exist |  |
| Global or local? | assumed Global |  |
| Belt speed | 0.07 m/s, measured off a video |  |
| Belt run time per move | unknown |  |
| Pause partway along the belt | none seen in the video |  |
| Belt sensors | none — timers only |  |
| Dobot M1 Pro address | 192.168.1.6 |  |
| Pro 600 address | 192.168.1.159, port 5001 |  |
| Pro 600 remote server on? | assumed yes |  |
| Joint directions, both arms | assumed same as my model |  |

If he can only do one thing Ask for the CCW project archive — the .ccwarc file. It contains the variable list, the data types and the ladder logic in one attachment, and it answers items 1.3, 1.4, 1.5 and most of 1.6 at once.

Questions

Seven questions about the cell

These are the ones I'd like answered directly, in plain words. Type into the grey boxes. Short answers are fine; “I don't know” is a useful answer too.

■  The three marked in this colour change how I build it, so they're the ones I'd like first.

1.  Is there a network switch at the bench that the PLC and both robots plug into? Or is each thing wired on its own?

My laptop has to sit on the same network as them to read anything at all.

There is a network switch where the 2 robots, PLC, and laptop connect

2.  When you program the PLC, do you plug in an Ethernet cable or a USB cable?

If it's always USB, the PLC's network side may never have been switched on. That's not a problem — it just becomes a small setup job, and it's better to know now than on the day.

Ethernet

3.  What turns the vacuum on — the PLC, or the Pro 600 robot's own outputs?

Changes how I build it. My code assumes the PLC controls it. If the robot does, I've been planning to read the wrong device.

The PLC

4.  Do the PLC and the two robots talk to each other over the network, or are they joined by plain wires — one sends a “go” signal and the other reacts?

Changes how I build it. It decides whether the PLC knows what the robots are doing, or whether I have to read all three separately and line them up myself.

The PLC knows what the robots are doing, once the robots finish their tasks, they send a message to the PLC through MODBUS TCP and the PLC activate the next task

5.  Who starts a cycle — somebody pressing a button, the PLC on its own, or a person hitting play on a robot pendant?

Tells me what signal to watch for as “a new cycle has begun”, so the simulation starts at the same moment the machine does.

I botton in the PLC

6.  Are there any sensors on the cell at all — something that detects the holder arriving, or that a wafer is present?

Changes how I build it. Right now I assume there are none and work out where the holder is by counting seconds from when the belt starts. A real sensor would be far more accurate, and I'd rather read it than guess.

No sensors yet, will add proximity sensors in the future

7.  Does the belt ever run backwards, or does the holder always travel one way and get carried back by hand?

In my simulation the belt returns the holder by itself. If the real one doesn't, the two stop matching after the first cycle.

Right now only travels one way but in the future will probably run backwards

Read-only throughout: the simulation observes the cell and never commands it. Nothing on this list changes how the machine runs, with the single exception of item 1.3, which may add mirrored Global Variables to the PLC program.